ENGINEERING TEAM INTERNAL BRIEFING Teltonika Hardware & Telematics Platform Architecture Engineering Team Technical Briefing

April 2026 ¦ Version 1.0 ¦ CONFIDENTIAL — Internal Use Only

PURPOSE OF THIS DOCUMENT This briefing equips the Kasper engineering team with everything needed to begin research and development on our GPS telematics platform — from the physical hardware installed on machines, to the raw data format those devices transmit, to the full software stack required to transform that data into actionable business intelligence. This is a zero-to-one briefing. Assume no prior knowledge of telematics hardware or protocols.

# Section 1 — Hardware Overview: The Two Devices

Teltonika is the industry-standard choice for vehicle and heavy equipment telematics across the GCC and globally. They manufacture rugged, proven devices specifically rated for harsh outdoor environments — critical for construction sites in UAE heat. Kasper will deploy two device tiers depending on the asset type.

## 1.1  Device Comparison at a Glance

| Attribute | FMC920 (Basic) | FMC130 (Professional — Recommended) |
| --- | --- | --- |
| Category | Basic Tracker | Advanced Tracker |
| Primary Use Case | Light vehicles, trucks, support equipment | Heavy machinery: excavators, cranes, generators, loaders |
| 4G LTE | Yes — Cat 1 + 2G fallback | Yes — Cat 1 + 3G + 2G fallback |
| GNSS Systems | GPS, GLONASS, GALILEO, BEIDOU, SBAS, QZSS | GPS, GLONASS, GALILEO, BEIDOU, SBAS |
| Position Accuracy | < 2.5 m CEP | < 2.5 m CEP |
| GNSS Sensitivity | -165 dBm | -165 dBm |
| Internal Memory | 128MB (approx. 422,400 records offline) | 128MB (approx. 422,400 records offline) |
| Backup Battery | 170 mAh Li-Ion (device survives power cut) | 170 mAh Li-Ion |
| Bluetooth | 4.0 LE — connects to BLE sensors/dongles | 4.0 LE — connects to BLE sensors/dongles |
| Digital Inputs (DIN) | 1 | 3 (including 1 Negative Input + 2 Impulse Inputs) |
| Analog Inputs (AIN) | 1 | 2 |
| Digital Outputs (DOUT) | 1 | 3 (relay control, immobilizer, buzzers) |
| CAN Bus Support | Via external OBD-II dongle only | Native LVCAN interface (direct CAN wiring) |
| Impulse Input (Fuel Flow) | No | Yes — purpose-built for high-precision fuel flow meters |
| 1-Wire Support | No | Yes — temperature sensors + iButton operator ID |
| Negative Input | No | Yes — simplifies accessory wiring (doors, seat belts) |
| Ingress Protection | IP54 (dust/splash proof) | IP41 (indoor/cabin mount) |
| Operating Temperature | -40°C to +85°C | -20°C to +85°C |
| Weight | 54g | 55g |
| Dimensions | 79 x 43 x 12 mm | Small form factor — fits in tight spaces |
| Power Input | 10–30V DC | 10–30V DC |
| Overvoltage Protection | Up to 90V impulse (5ms) | Up to 90V impulse (5ms) |
| TRA / UAE Certified | Yes (CITC, TRA, CE/RED, E-Mark) | Yes (regional variants available) |
| Kasper Recommendation | Support fleet, logistics trucks | All heavy equipment — PRIMARY CHOICE |

## 1.2  FMC920 — Full Technical Specification

### Connectivity

- LTE Cat 1 with 2G fallback — future-proof for UAE's network evolution

- LTE FDD Bands: B1/B3/B7/B8/B20/B28 (covers all UAE carriers: du, Etisalat/e&)

- Data rates: LTE up to 10 Mbps down / 5 Mbps up; GPRS up to 85.6 Kbps fallback

- GNSS: GPS + GLONASS + GALILEO + BEIDOU + SBAS — maximum satellite availability in desert terrain

- Bluetooth 4.0 LE for OBD-II dongles, temperature/humidity beacons, driver ID beacons

### Interface

- 1x Digital Input (DIN1) — ignition detection, door sensor, alarm button

- 1x Digital Output (DOUT1) — engine immobilizer relay, external LED

- 1x Analog Input (AIN1) — fuel level sensor, temperature probe, liquid level

- Micro-USB for local configuration via Teltonika Configurator software

### Scenarios / Built-in Logic

- Green Driving — detects harsh acceleration, braking, cornering via accelerometer

- Over-Speeding Detection — configurable speed threshold alerts

- Jamming Detection — alerts when GPS/GSM signal is actively blocked

- Crash Detection — accelerometer-based impact sensing

- Towing Detection — triggers when vehicle moves without ignition

- Unplug Detection — alerts when device is removed from power

- Auto Geofence — automatically sets a geofence around parked position

- Excessive Idling Detection — tracks engine-on without movement

- GNSS Fuel Counter — software-estimated fuel burn from movement data

- Trip monitoring — automatic trip start/end detection

### Power & Physical

- Power: 10-30V DC — compatible with all vehicle electrical systems

- Ultra Deep Sleep: < 2 mA — near-zero drain when asset parked for days

- IP54 rated — safe for engine bay, external chassis mounting

- Certifications: CE/RED, E-Mark, CITC, TRA, RCM, RoHS, REACH

## 1.3  FMC130 — Full Technical Specification (Primary Device for Kasper)

### Connectivity

- 4G LTE Cat 1 + 3G (UMTS) + 2G fallback — three-layer network redundancy

- GNSS: GPS + GLONASS + GALILEO + BEIDOU — -165 dBm sensitivity

- Bluetooth 4.0 LE for wireless sensors and OBD-II dongles

- Internal flash: 128MB (stores up to 422,400 records when offline — dumps when reconnected)

### Critical Interfaces for Heavy Equipment

- LVCAN interface — native CAN bus RX/TX lines for direct connection to machine ECU

- 3x Digital Inputs — ignition, door sensor, PTO (power take-off), hydraulic circuit status

- 1x Negative Input (DIN2) — simplified accessory wiring without dedicated ground lines

- 2x Impulse Inputs (DIN1, DIN2) — purpose-built for fuel flow meters (pulse counting = exact litres consumed)

- 2x Analog Inputs — 0-10V range, connects to fuel level probes, hydraulic pressure sensors

- 3x Digital Outputs — engine immobilizer relay, beacon light, buzzer

- 1-Wire temperature sensor — ambient, engine compartment, or cargo temperature

- 1-Wire iButton — operator ID authentication (machine will not start without registered key)

### Power & Environmental

- Power input: 10-30V DC (with overvoltage protection to 90V impulse)

- Operating temperature: -20°C to +85°C without battery (survives UAE summer equipment temps)

- Ultra Deep Sleep: < 3 mA; Nominal with no load: < 33 mA

- Internal fuse: 3A, 125V — self-protective against wiring faults

- Weight: 55g — small enough to conceal inside equipment cab or control panel

## 1.4  CAN Bus — The Most Important Concept for Heavy Equipment

What is CAN Bus? (Start Here)

CAN Bus (Controller Area Network) is the internal computer network inside every modern vehicle or machine.

Every sensor in a Caterpillar excavator, JCB loader, or Komatsu grader — the engine, fuel system, hydraulics,

transmission — all talk to each other over the CAN bus. It is the machine's nervous system.

The FMC130 can tap into this network and READ all of that data in real time.

Without CAN access: you get GPS position only.

With CAN access: you get position + engine hours + fuel level + RPM + fault codes + hydraulic status + more.

This is the difference between basic tracking and genuine telematics intelligence.

### CAN Protocols to Know

- J1939 — the heavy equipment and truck standard. Used by Caterpillar, Komatsu, JCB, Volvo CE, XCMG, Liugong. This is the protocol the FMC130 reads natively via its LVCAN interface.

- OBDII — the light vehicle standard (cars, pickups, light trucks). The FMC920 reads this via an external Bluetooth dongle.

- J1708/J1587 — older heavy equipment protocol (pre-2000 machines). Less common but some legacy Gulf fleet machines may use it.

- FMS (Fleet Management System) — a standardised CAN gateway in European trucks. Accessible via a dedicated 9-pin FMS connector under the dashboard.

### What CAN Data Gives You — AVL Parameter Examples

| AVL Parameter ID | Data Point | Business Use |
| --- | --- | --- |
| AVL ID 12 | Engine RPM | Detect operating vs. idle vs. off. Bill by engine-on hours. |
| AVL ID 16 | Total Mileage / Odometer | Virtual odometer — tracks exact machine distance |
| AVL ID 66 | External Voltage | Detect if machine power was cut — theft signal |
| AVL ID 67 | Battery Voltage | Machine battery health monitoring |
| AVL ID 68 | Fuel Level (%) | Real-time fuel level from tank sensor via CAN |
| AVL ID 72 | Engine Coolant Temperature | Predictive maintenance — overheat alerts |
| AVL ID 181 | Fuel Rate (L/hr) | Live fuel burn rate — catch abnormal consumption |
| AVL ID 182 | Total Fuel Used | Cumulative fuel used — essential for cost tracking |
| AVL ID 253 | Engine Total Hours | Maintenance scheduling (service every N hours) |
| AVL ID 10493 | DTC Fault Code (J1939 SPN) | Machine fault detection — push alert to operator |
| AVL ID 10494 | DTC Failure Mode (J1939 FMI) | Fault severity: active / pending / historical |
| AVL ID 525 | Engine Load % | Is the machine working hard or idling? Utilisation metric. |
| AVL ID N/A | Hydraulic Status (Front) | Is the boom/arm/bucket actively operating? |
| Custom | Accelerator Pedal Position % | Operator driving behaviour on heavy machines |

## 1.5  Recommended Accessories

| Accessory | Model / Type | Purpose | Applies To |
| --- | --- | --- | --- |
| CAN Adapter | Teltonika CAN-CONTROL / LVCAN adapter | Bridges FMC130 to vehicle CAN bus without splicing wires | FMC130 |
| iButton Reader | Teltonika iButton / Dallas DS1990A | Operator ID — machine won't start without registered key | FMC130 |
| Bluetooth OBD-II Dongle | Teltonika EYE OBD / Howen | Adds CAN data to FMC920 via Bluetooth without wiring | FMC920 |
| Fuel Level Sensor (Capacitive) | Technoton or LLS series | Ultrasonic fuel probe inserted into tank — high accuracy | FMC130 (AIN) |
| Fuel Flow Meter (Impulse) | Piusi / Aquametro series | Inline flow meter — pulse counting via DIN impulse input | FMC130 (Impulse) |
| Temperature Sensor | Teltonika 1-Wire TSP01 | Engine bay or cargo temperature monitoring | FMC130 |
| EYE Bluetooth Sensor | Teltonika EYE (BLE beacon) | Proximity/door open detection via Bluetooth | Both |
| External GNSS Antenna | Teltonika 003R-00253 | Improves GPS fix in metal-caged machinery cabs | FMC130 |
| Engine Immobilizer Relay | Standard 12V/24V relay (20A) | Wired to DOUT — remote engine disable via platform | FMC130 |
| Dashcam (AI) | Teltonika DashCam DCI200 | Driver behaviour video evidence with AI event tagging | FMC130 |

# Section 2 — How the Data Flows: From Device to Database

This section describes the complete technical data journey — from the moment a Teltonika device wakes up and generates a location record, to the moment it lands in a database that your dashboards can query.

## 2.1  Overview of the Pipeline

The 6-Stage Data Pipeline

Stage 1: Hardware generates data packet (AVL record) inside the device

Stage 2: Device transmits raw binary packet over TCP/UDP to your server IP:Port

Stage 3: TCP/UDP Listener server receives the packet and sends acknowledgement

Stage 4: Parser decodes the binary Codec 8/8E format into human-readable JSON

Stage 5: Message broker (Kafka/RabbitMQ) queues JSON for processing

Stage 6: Time-series database stores the records for querying by your dashboard

## 2.2  Stage 1 — AVL Record Generation on the Device

An AVL (Automatic Vehicle Location) record is what Teltonika calls a single data snapshot. It is generated based on rules you configure:

- Time interval — e.g., 'record every 10 seconds when moving'

- Distance interval — e.g., 'record every 50 metres of travel'

- Angle change — e.g., 'record when heading changes by more than 10 degrees'

- I/O event — e.g., 'record immediately when ignition state changes'

- Geofence event — e.g., 'record when device enters or exits a defined zone'

Each AVL record contains: a UNIX timestamp, GPS coordinates (lat/lon/altitude/speed/heading/satellites), and a payload of all the I/O parameter values you have configured (fuel level, RPM, DINs, engine hours, etc.).

## 2.3  Stage 2 — The Transmission Format (Critical for Your Dev Team)

IMPORTANT: This Is Not JSON. This Is Binary Hex.

Teltonika devices do NOT send neat JSON over HTTP like a REST API.

They transmit raw binary data encoded in hexadecimal over a persistent TCP or UDP socket.

Example of what arrives at your server port:

000000000000002A8E010000016B40D8EA30010000000000000000000000000000000105021503010101425E0F01F10000601A014E0000000000000000010000C7CF

Your dev team must decode this hex string using Teltonika's Codec 8 / Codec 8 Extended protocol specification.

This is the central technical challenge of the entire platform build.

### Codec 8 vs Codec 8 Extended

| Property | Codec 8 | Codec 8 Extended |
| --- | --- | --- |
| AVL Parameter IDs | 1 byte (IDs 0–255) | 2 bytes (IDs 0–65535) — needed for advanced CAN data |
| Supported Parameters | Basic GPS + simple I/O | Full CAN data, fuel, engine hours, fault codes |
| Recommended For | FMC920 basic tracking | FMC130 heavy equipment with CAN — use this |
| Codec ID Byte | 0x08 | 0x8E (decimal 142) |
| Max Records per Packet | 255 | 255 |

## 2.4  Stage 3 — The TCP/UDP Listener Server

Your backend team must build (or deploy) a persistent socket server. This is not a standard HTTP web server. It is a raw TCP/UDP server that:

- Listens on a specific port (default Teltonika port: 5027 for TCP)

- Keeps the socket connection open — Teltonika devices maintain persistent connections

- Receives binary data packets from potentially thousands of devices simultaneously

- Sends back a 4-byte acknowledgement to confirm receipt (device waits for this before clearing its buffer)

- Handles disconnections and reconnections gracefully

Language recommendation for the listener: Go (best for high concurrency), Node.js (fastest to prototype), or Python (asyncio). Go is the production recommendation for 1,000+ devices.

Shortcut Available — Use Traccar for Phase 1

Traccar (traccar.org) is a free, open-source telematics backend that already has a complete, production-tested

Teltonika parser and TCP listener built in. It can receive FMC130/FMC920 data out of the box and expose it

via REST API endpoints your dashboard can query.

Phase 1 Recommendation: Deploy Traccar on a UAE cloud instance (AWS Bahrain / Azure UAE North) to skip

3–4 months of custom parser development. Build your custom platform on top of Traccar's API.

Traccar is also what powers many commercial telematics platforms — it is production-grade, not a toy.

## 2.5  Stage 4 — Decoding Codec 8 Extended (The Parser)

If building a custom parser (Phase 2 or when outgrowing Traccar), this is the exact packet structure your team must decode:

| Bytes | Field | Description |
| --- | --- | --- |
| 4 | Preamble | Always 0x00000000 — marks start of a valid packet |
| 4 | Data Field Length | Length of the following data in bytes |
| 1 | Codec ID | 0x8E = Codec 8 Extended (most common for FMC130) |
| 1 | Number of Data 1 | How many AVL records follow in this packet |
| N | AVL Data Records | Repeating block: timestamp + GPS + I/O parameters (see below) |
| 1 | Number of Data 2 | Must match Number of Data 1 — validation check |
| 4 | CRC-16/IBM Checksum | Error detection — validate before processing |

### AVL Record Structure (Inside Each Record)

| Bytes | Field | Description |
| --- | --- | --- |
| 8 | Timestamp | UNIX time in milliseconds — divide by 1000 for seconds |
| 1 | Priority | 0=Low, 1=High, 2=Panic — panic triggers immediate send |
| 4 | Longitude | Multiply by 10^-7 to get decimal degrees |
| 4 | Latitude | Multiply by 10^-7 to get decimal degrees |
| 2 | Altitude | Metres above sea level |
| 2 | Angle | Heading in degrees (0=North, 90=East, 180=South, 270=West) |
| 1 | Satellites | Number of GPS satellites used for this fix |
| 2 | Speed | Speed in km/h |
| Variable | I/O Element Block | All configured parameters (fuel, RPM, engine hours, etc.) |

The I/O Element Block itself has a nested structure: it lists 1-byte, 2-byte, 4-byte, and 8-byte parameters separately. Each parameter has an ID (2 bytes for Codec 8E) and its value. Your parser must loop through each group and decode accordingly.

Open-source starting points: search GitHub for 'teltonika codec parser python' or 'teltonika codec8 node'. Multiple production-quality parsers exist in Node.js, Python, and Go. Never write one from scratch without reviewing existing implementations first.

## 2.6  Stage 5 — Message Broker (Kafka / RabbitMQ)

Once a packet is decoded into JSON, it must NOT be written directly to the database. At scale:

- 1,000 devices x 1 packet per 10 seconds = 6,000 database writes per minute

- Direct DB writes will cause lock contention, query slowdowns, and eventual crashes

- A message broker (Apache Kafka or RabbitMQ) acts as a buffer — absorbs the fire hose and lets consumers (database writers, alert engines, analytics pipelines) process at their own pace

Phase 1 shortcut: If using Traccar, it handles this internally. Only needed when building custom pipeline at scale (Phase 2).

## 2.7  Stage 6 — Time-Series Database Storage

| Database | Best For | Notes |
| --- | --- | --- |
| TimescaleDB | Production telematics — recommended | PostgreSQL extension — familiar SQL, excellent for time-series, supports PostGIS for spatial queries |
| InfluxDB | Metrics and sensor data | Excellent time-series performance, different query language (Flux), no SQL |
| ClickHouse | High-volume analytics (100M+ records) | Columnar store — blazing fast aggregations, complex to operate |
| PostgreSQL + PostGIS | Phase 1 / small scale | Standard relational DB + geospatial extension — sufficient for <500 devices |

Critical: Do NOT use a standard relational database (MySQL, vanilla Postgres) for raw telemetry at scale. At 1,000 devices sending every 10 seconds, you will generate ~86 million records per day. Only a TSDB can handle this economically.

## 2.8  What Decoded JSON Looks Like (Your API Output)

{ "device_id": "352625333222111", "timestamp": 1712000000, "position.latitude": 25.2048, "position.longitude": 55.2708, "position.altitude": 12, "position.speed": 0, "position.heading": 270, "position.satellites": 14, "engine.rpm": 1450, "engine.hours": 4821.5, "engine.coolant_temp": 87, "fuel.level_percent": 62, "fuel.rate_lph": 8.4, "fuel.total_used_litres": 12483.2, "engine.load_percent": 73, "ignition": true, "movement": false, "dtc_codes": ["J1939-SPN:110/FMI:3"], "operator_ibutton": "0A1B2C3D4E5F0001" }

# Section 3 — Full Feature Backlog for the Kasper Platform

Below is the complete feature set your platform should eventually deliver, organised by category with technical complexity ratings. Your tech team should treat this as the product backlog against which Phase 1, Phase 2, and Phase 3 sprints are planned.

## 3.1  Asset Visibility (Location & Mapping)

| Feature | Description | Tech Required | Complexity |
| --- | --- | --- | --- |
| Real-time Map View | Live positions of all assets on a Mapbox/Google Maps dashboard. Updates every 5-30s. | WebSocket push from server to frontend; Mapbox GL JS or Google Maps API | 🟢 Low |
| Historical Route Playback | Replay any asset's exact route for any date/time range as animated track on map | TSDB time-range query + frontend animation player | 🟢 Low |
| Multi-Site Asset Grouping | Group assets by construction site, client, or project — filter map view | Standard DB tagging + frontend filter logic | 🟢 Low |
| Asset Status Panel | At-a-glance status: online/offline, ignition on/off, current speed, last seen | Aggregation query on latest record per device | 🟢 Low |
| Offline Alert | Push notification if device has not reported for > N minutes (configurable) | Cron job checking last_seen timestamp per device | 🟢 Low |
| Address Lookup (Reverse Geocoding) | Convert lat/lon to human-readable address. 'Al Quoz Industrial Area, Dubai' | Google Maps Geocoding API or HERE API | 🟢 Low |

## 3.2  Utilisation & Billing Intelligence

| Feature | Description | Tech Required | Complexity |
| --- | --- | --- | --- |
| Engine Hours Tracking | Count total hours engine was ON (ignition=true). This is how equipment rental is billed. | CAN AVL ID 253 (or ignition state accumulation in TSDB) | 🟡 Medium |
| Active vs Idle Hours | Split engine hours into 'Working' (moving + load) vs 'Idle' (engine on, no movement) | RPM threshold + movement state logic in analytics layer | 🟡 Medium |
| Daily Utilisation Report | Per-asset daily breakdown: total ON hours, idle %, distance, sites visited | TSDB aggregation + PDF/email report generation | 🟡 Medium |
| Billing Automation | Auto-calculate rental invoice based on actual engine hours at contracted rate | Engine hours API → billing engine → Zoho Books integration | 🟡 Medium |
| Utilisation Benchmarking | Compare utilisation % across fleet. Flag underperforming assets for redeployment. | Statistical aggregation across asset group | 🟡 Medium |
| Machine Score | Composite score per asset: utilisation %, fuel efficiency, fault frequency, idle% | Weighted formula across multiple TSDB metrics | 🔴 High |

## 3.3  Geofencing & Site Management

| Feature | Description | Tech Required | Complexity |
| --- | --- | --- | --- |
| Draw Site Geofences | Admin draws polygon or circle around construction site on map | PostGIS polygon storage; Mapbox Draw plugin for frontend | 🟡 Medium |
| Geofence Entry/Exit Alerts | SMS/push/email when any asset enters or leaves a designated zone | PostGIS ST_Contains() spatial query on each new record; MSG91/Twilio for SMS | 🟡 Medium |
| After-Hours Movement Alert | Alert when equipment moves outside of configured working hours | Timestamp comparison against configurable schedule per site | 🟡 Medium |
| Site Dwell Report | How long was each asset on-site vs in transit vs at depot? | Zone state tracking in TSDB + aggregation query | 🟡 Medium |
| Multi-Site Asset Dispatch | See which assets are available (at depot) vs deployed (on site) | Geofence state + current zone field on asset record | 🟡 Medium |
| Unauthorized Zone Alert | Alert when asset enters a non-permitted zone (e.g., competitor's site) | Exclusion zone polygon; same PostGIS logic as entry/exit | 🟡 Medium |

## 3.4  Fuel Monitoring (Revenue-Critical Feature)

Why Fuel Is The Most Important Feature

Fuel theft and excess fuel consumption are the #1 cost driver for heavy equipment operators in the GCC.

A single Caterpillar 320 excavator burns 12-18 litres/hour. At AED 3.00/litre, that is AED 36-54/hour.

Operators regularly siphon fuel or falsify records. Your platform catching even one theft event per month

per machine covers your GPS subscription cost for the entire year.

This feature is your most powerful selling point to vendors — lead with it.

| Method | Hardware | Accuracy | Best For |
| --- | --- | --- | --- |
| CAN Bus Fuel Level | FMC130 + CAN adapter | Good (±5%). Data direct from ECU, no extra hardware. | Caterpillar, Komatsu, JCB (modern machines) |
| Analog Fuel Level Sensor (LLS) | FMC130 AIN + capacitive probe | Very good (±2-3%). Reads actual tank depth via ultrasonic probe. | Any machine — probe inserted into tank cap |
| Fuel Flow Meter (Impulse) | FMC130 Impulse DIN + Piusi flow meter | Excellent (±0.5%). Inline meter counts exact litres flowing. | High-value machines, generators, cranes |
| OBD-II Bluetooth Dongle | FMC920 + EYE OBD | Moderate (±8-10%). Software estimated. | Light trucks, support vehicles only |

### Fuel Features to Build

- Real-time fuel level gauge per machine on dashboard

- Fuel level history graph (timeline view — spot sudden drops = theft or leak)

- Refuel event detection — sudden fuel level increase = refuel (log timestamp, location, litres added)

- Fuel drain alert — fuel level drops without engine running = theft

- Fuel efficiency metric — litres per engine hour (L/EH) per machine, benchmark across fleet

- Daily fuel consumption report per asset and per site

- Fuel theft incident log with GPS coordinates, timestamp, and before/after level

## 3.5  Engine Health & Predictive Maintenance

| Feature | Description | Complexity |
| --- | --- | --- |
| Engine Hours Counter | Cumulative hours since machine manufactured (from CAN) or since tracker installed | 🟢 Low — direct CAN parameter |
| Maintenance Schedule | Set service intervals: 'Oil change every 250 engine hours.' Alert when due in 50 hours. | 🟡 Medium — CRUD logic against engine hours total |
| Service Log | Record every service: date, hours at service, work done, cost. History per machine. | 🟡 Medium — standard data entry + document storage |
| Fault Code (DTC) Alerts | J1939 fault codes pushed to dashboard and operator SMS in real time | 🔴 High — J1939 DTC decoding library required |
| Engine Temperature Monitoring | Alert when coolant temp exceeds threshold (machine overheating in UAE heat) | 🟡 Medium — threshold alert on CAN parameter |
| Battery Voltage Monitoring | Alert if vehicle battery drops below 11.5V — prevent no-start situations | 🟢 Low — AVL ID 67 threshold alert |
| Predictive Maintenance Model | ML model predicts failure probability based on historical fault + usage patterns | 🔴🔴 Very High — Phase 3 only, requires historical data |

## 3.6  Operator Management & Access Control

| Feature | Description | Complexity |
| --- | --- | --- |
| iButton Operator Authentication | Machine will not start unless a registered operator taps their iButton. FMC130 reads the key and logs the operator ID. | 🔴 High — requires DOUT relay wiring + iButton database |
| Operator Trip Assignment | Each trip automatically assigned to the operator who authenticated at start | 🟡 Medium — join operator ID with trip records |
| Operator Behaviour Scoring | Score each operator: harsh braking, harsh cornering, harsh acceleration, overspeeding | 🟡 Medium — accelerometer AVL parameters + threshold logic |
| Operator Hours Tracking | Total hours worked per operator (iButton session duration) | 🟡 Medium — session start/end from iButton events |
| Unauthorized Operator Alert | Alert when an unregistered iButton key is used (or machine started without key) | 🟡 Medium — whitelist check against registered keys |
| Operator Report Card | Weekly/monthly PDF report per operator with all behaviour metrics | 🟡 Medium — aggregation + PDF generation |

## 3.7  Remote Control & Commands

| Feature | Description | Complexity | Hardware |
| --- | --- | --- | --- |
| Remote Engine Immobilizer | Platform sends command → FMC130 DOUT activates relay → machine cannot start | 🔴 High — two-way communication, DOUT wiring, safety protocols required | FMC130 DOUT + relay |
| Remote Configuration Push | Update device settings (tracking interval, geofence) without physical access | 🟡 Medium — Teltonika FOTA WEB or custom GPRS command API | Both |
| Remote Firmware Update (FOTA) | Push firmware updates to all devices over-the-air from Teltonika FOTA portal | 🟢 Low — Teltonika provides FOTA WEB platform | Both |
| SMS Command Fallback | If GPRS unavailable, send SMS to device to request position or trigger output | 🟡 Medium — Teltonika supports predefined SMS command set | Both |
| Beacon / Alert Output | Trigger a buzzer or warning light on the machine from the platform (safety alert) | 🟡 Medium — DOUT command + buzzer wiring | FMC130 DOUT |

## 3.8  GPS-as-a-Service Layer (Kasper Revenue Stream)

This is the Layer 2 revenue model — charging vendor operators a monthly subscription for access to the telematics dashboard for their own machines. The following features define the vendor-facing product.

| Feature | Description | Complexity |
| --- | --- | --- |
| Vendor Portal (White-label option) | Dedicated login for each equipment vendor to see only their assets | 🟡 Medium — multi-tenant role-based access control |
| Self-Service Alerts | Vendor can configure their own alert rules (fuel threshold, zone exits) from their portal | 🟡 Medium — alert rule builder UI + per-tenant rule engine |
| Asset Health Dashboard | Vendor sees engine hours, fuel, faults, maintenance schedule for their machines | 🟡 Medium — filtered views of existing data per vendor |
| Downloadable Reports | Vendor downloads monthly PDF report: utilisation, fuel, maintenance, operator behaviour | 🟡 Medium — scheduled report generation + email delivery |
| API Access (Advanced Tier) | Senior vendors pay for raw API access to integrate with their own ERP/systems | 🔴 High — API key management, rate limiting, documentation |
| Fleet Benchmarking | Show vendor how their machines compare to fleet average (anonymized) | 🔴 High — requires sufficient fleet data for meaningful benchmarks |

# Section 4 — Reality Check: Where the Hard Problems Live

Building the dashboard is the straightforward part. The hidden complexity in telematics lives at the hardware, data, and physical installation layers. This section is a candid briefing for your tech lead on what will actually cause delays.

| Challenge | Why It's Hard | Mitigation |
| --- | --- | --- |
| 1. Hardware Installation on Heavy Equipment | Hardwiring an FMC130 into the CAN bus of a Caterpillar 320 excavator is NOT like plugging in an OBD dongle. It requires a licensed auto-electrician or heavy equipment technician who understands the machine's electrical schematic. Incorrect wiring can trigger ECU faults or void OEM warranty. In UAE, this service costs AED 500–1,500 per machine. This is your operational bottleneck, not your software. | Partner with 1-2 dedicated auto-electricians specialising in heavy equipment. Start with machines that have accessible FMS connectors. Use CAN adapters (not bare wire splicing) wherever possible. |
| 2. Parsing the Codec 8 Protocol | Writing a flawless Codec 8 / 8 Extended binary parser from scratch requires careful reading of the Teltonika protocol documentation and significant test coverage. One byte offset error corrupts all data silently. | Use an existing open-source parser (GitHub: 'teltonika-codec8-parser'). Or deploy Traccar which parses natively. Only build custom if Traccar proves insufficient at scale. |
| 3. CAN Mapping Differences by Brand | While J1939 is a standard, each OEM (Caterpillar, Komatsu, JCB, XCMG, Liugong) uses proprietary extended parameter mappings on top of the standard. Fuel level may appear on a different SPN for a Komatsu vs a CAT. You will need to test each brand separately. | Start your pilot with one machine brand. Document the AVL parameter mappings for that brand. Expand systematically. Check Teltonika's CAN parameter database — they maintain supported PGN/SPN mappings. |
| 4. GPS Signal in Construction Environments | Signals bounce off skyscrapers (urban canyon effect). Machines work underground or under canopies. Metal cabs attenuate GPS signal. These cause position drift, missing records, and false movement alerts. | Use external GNSS antenna on FMC130 for metal-cab machines. Configure devices to store records offline and backfill — Teltonika does this natively (up to 422,400 records in flash). |
| 5. Data Backfill Spikes | When a device comes back online after being underground or offline, it dumps all stored records at once. A single machine could send 10,000 records in one burst. If your pipeline writes directly to DB, this crashes it. | Kafka or RabbitMQ message broker absorbs bursts. Database writes process at constant rate regardless of ingestion spikes. Critical for production. |
| 6. Data Volume at Scale | 1,000 machines x 1 record per 10 seconds = 8.64 million DB writes per day. At 50 fields per record, that is 432 million field values per day. Standard Postgres will not survive this without proper architecture. | TimescaleDB with automatic chunk partitioning by time. Appropriate indexing on (device_id, timestamp). Cloud-managed instance (AWS RDS or Timescale Cloud) for Phase 1 to avoid self-managing. |
| 7. Two-Way Command Security | When your platform can remotely disable an engine (DOUT immobilizer), you have safety liability. An unauthorized command or software bug that immobilizes a machine in an active work zone is dangerous. | Require two-factor confirmation for immobilizer commands. Log all command issuance with user ID and timestamp. Build a mandatory safety check: immobilizer only permitted when machine is stationary (speed = 0). |

# Section 5 — Recommended Technology Stack

| Layer | Phase 1 (MVP) | Phase 2 (Scale) | Notes |
| --- | --- | --- | --- |
| TCP Listener | Traccar (open-source, self-hosted) | Custom Go TCP server | Traccar available free on GitHub — deploy on EC2/VPS |
| Protocol Parser | Traccar built-in (Teltonika protocol) | Custom Codec 8E parser in Go | Traccar handles FMC130 and FMC920 natively |
| Message Broker | Not required (Traccar buffers internally) | Apache Kafka | Add Kafka when fleet exceeds ~200 devices |
| Database | PostgreSQL + PostGIS (Traccar default) | TimescaleDB | TimescaleDB is Postgres-compatible — migration is straightforward |
| Backend API | Traccar REST API | Custom Node.js / Go API | Traccar exposes full REST API — build frontend against it directly in Phase 1 |
| Frontend Dashboard | Softr or Retool (no-code Phase 1) | React + Mapbox GL JS | Retool or Softr can connect to Traccar API in days |
| Maps | Traccar built-in map (OpenStreetMap) | Mapbox GL JS | Mapbox pricing: free up to 50,000 loads/month |
| Alerts / Notifications | Traccar built-in email/SMS alerts | Custom rule engine + MSG91 | Traccar supports SMS via gateway integration |
| Cloud | AWS Bahrain (me-south-1) or Azure UAE North | Same — scale vertically then horizontally | Low latency to UAE devices. Start with t3.large (~$70/month) |
| Device Configuration | Teltonika Configurator desktop app (free) | FOTA WEB (Teltonika cloud — paid per device) | FOTA WEB automates firmware/config push across all devices remotely |

## 5.1  Phase 1 Critical Path — First 60 Days

| Week | Milestone | Responsible |
| --- | --- | --- |
| Week 1–2 | Procure 2x FMC130 + 1x FMC920 test units from UAE distributor. Insert UAE SIM cards. | Ops + Tech |
| Week 2–3 | Stand up Traccar on AWS Bahrain EC2 instance. Configure devices to point to server IP:Port. | Tech |
| Week 3–4 | Install FMC130 on 1 test machine (with electrician). Verify data appearing in Traccar dashboard. | Tech + Ops |
| Week 4–5 | Configure CAN parameters on FMC130 for test machine brand. Verify fuel/RPM/hours in Traccar. | Tech |
| Week 5–6 | Build basic Retool/Softr dashboard consuming Traccar REST API. Asset map + fuel gauge + engine hours. | Tech |
| Week 6–8 | Install on 5–10 vendor fleet machines. Validate all alerts and reports. Iterate on dashboard. | Tech + Ops |
| Week 8+ | Pilot live with 1–2 paying vendor subscribers on GPS-as-a-Service tier. | Business + Tech |

## 5.2  Key Reference Resources for Your Tech Team

| Resource | URL | What It's For |
| --- | --- | --- |
| Teltonika Wiki — Main Index | wiki.teltonika-gps.com/view/Main_Page | Master index for all device documentation, protocols, and guides |
| Codec 8 / 8E / 12 / 14 Protocol Spec | wiki.teltonika-gps.com/view/Codec | THE primary reference for building your TCP parser — full packet structure |
| Data Sending Protocols | wiki.teltonika-gps.com/view/Teltonika_Data_Sending_Protocols | TCP/UDP structure, packet format, acknowledgement logic |
| Traccar (open-source backend) | github.com/traccar/traccar | Free Phase 1 backend — already parses FMC130/FMC920 natively |
| Traccar Documentation | www.traccar.org/documentation/ | Setup, REST API reference, alert configuration |
| Teltonika Community Forum | community.teltonika.lt | Real-world installer Q&A, UAE-specific issues, troubleshooting |
| Flespi (managed parsing API) | flespi.io | Commercial alternative to Traccar — managed, paid per device |
| TimescaleDB Documentation | docs.timescale.com | Time-series database for telemetry at scale |
| PostGIS Documentation | postgis.net/documentation/ | Spatial query extension for geofencing (ST_Contains, polygons) |
| Mapbox GL JS | docs.mapbox.com/mapbox-gl-js/ | Mapping SDK for the frontend dashboard |

Summary: What Your Tech Team Must Do First

1. Read: FMC130 Wiki General Description + Codec page + AVL Parameter IDs — in that order.

2. Procure: 2x FMC130 test units from a UAE distributor. Insert UAE SIM cards (du or Etisalat data plan).

3. Deploy: Traccar server on AWS Bahrain (me-south-1). Configure devices to point to server IP:Port 5027.

4. Test Loop: Stand up 1 device → receive 1 AVL record → decode it → see lat/lon in Traccar dashboard.

Mastering this single data loop is the foundation of everything else.

5. CAN Test: Install FMC130 on 1 machine with electrician → verify engine hours + fuel via CAN.

6. Build: Start dashboard on top of Traccar REST API — do not build a custom backend in Phase 1.

# Section 6 — Official Teltonika Documentation & Download Links

All links below are direct from Teltonika's official wiki (wiki.teltonika-gps.com) and product servers. These are the authoritative sources — not reseller sites or third-party mirrors. Bookmark these. Every question your team has during development will be answered somewhere in these pages.

Three Links to Open First — Before Anything Else

1. wiki.teltonika-gps.com/view/Codec

→ Full Codec 8 / Codec 8 Extended protocol spec. Required to build your data parser.

2. wiki.teltonika-gps.com/view/FMC130_Teltonika_Data_Sending_Parameters_ID

→ Master list of every AVL ID, what it means, units, and how to decode it.

3. wiki.teltonika-gps.com/images/5/55/QM-FMC130.pdf

→ Quick Manual PDF with wiring pinout diagram. The electrician needs this on-site.

## 6.1  FMC130 — Professional Device (Primary)

### Datasheets & Manuals

| Document | Direct URL | Description |
| --- | --- | --- |
| Datasheet PDF | wiki.teltonika-gps.com/images/7/7b/Datasheet-FMC130.pdf | Full technical spec sheet — electrical ratings, dimensions, certifications, feature list. Share this with procurement and electricians. |
| Quick Manual PDF (v1.7) | wiki.teltonika-gps.com/images/5/55/QM-FMC130.pdf | Installation guide, pinout table, SIM insertion, wiring diagram, LED status codes. The electrician's field reference. |
| Wiki Hub (all FMC130 pages) | wiki.teltonika-gps.com/view/FMC130 | Central index page linking to every FMC130 sub-document — start here when navigating. |
| General Description | wiki.teltonika-gps.com/view/FMC130_General_description | Detailed hardware specs: cellular bands, GNSS performance, battery info, input/output electrical characteristics, operating temperature ratings. |

### Configuration & Parameters

| Document | Direct URL | Description |
| --- | --- | --- |
| AVL Data Sending Parameters ID | wiki.teltonika-gps.com/view/FMC130_Teltonika_Data_Sending_Parameters_ID | THE master reference — every AVL ID number, parameter name, units, and data size. Essential for parser development and CAN data configuration. |
| I/O Settings Guide | wiki.teltonika-gps.com/view/FMC130_I/O_settings | How to enable and configure each I/O element (fuel, ignition, RPM, etc.), set priorities, and define event triggers. |
| Full Parameter List | wiki.teltonika-gps.com/view/FMC130_Parameter_list | Every configurable parameter in the device — tracking intervals, sleep modes, geofence logic, all settings. |
| Beacon Configuration | wiki.teltonika-gps.com/view/FMC130_Beacon_List | BLE beacon setup — for EYE sensors, operator proximity beacons, temperature/humidity BLE sensors. |

### Product & Downloads Page

| Resource | URL | Notes |
| --- | --- | --- |
| Official Product Page | www.teltonika-gps.com/products/trackers/advanced/fmc130 | Downloads hub for certifications, drivers, configurator, and ordering info. |
| ManualsLib Quick Manual | www.manualslib.com/manual/1820545/Teltonika-Fmc130.html | Indexed, searchable version of the Quick Manual — useful for quick lookups without downloading the PDF. |

## 6.2  FMC920 — Basic Device (Support Vehicles)

### Datasheets & Manuals

| Document | Direct URL | Description |
| --- | --- | --- |
| Datasheet PDF | wiki.teltonika-gps.com/images/2/22/DS-FMC920.pdf | Full technical spec sheet — electrical ratings, antenna specs, interface summary, scenario list, certifications. |
| Quick Manual PDF (v3.0 — latest) | wiki.teltonika-gps.com/images/7/75/QM-FMC920.pdf | Installation guide, wiring, Bluetooth pairing, LED codes, SMS configuration commands. |
| Quick Manual PDF (v2.9) | wiki.teltonika-gps.com/images/9/92/Quick-manual-FMC920.pdf | Previous version — useful if working with older firmware units. |
| Wiki Hub (all FMC920 pages) | wiki.teltonika-gps.com/view/FMC920 | Central index for all FMC920 documentation. |

### Configuration & Parameters

| Document | Direct URL | Description |
| --- | --- | --- |
| AVL Data Sending Parameters ID | wiki.teltonika-gps.com/view/FMC920_Teltonika_Data_Sending_Parameters_ID | Master AVL ID reference for FMC920 — note: subset of FMC130 parameters (no CAN IDs). |
| I/O Settings Guide | wiki.teltonika-gps.com/view/FMC920_I/O_settings | Configure ignition, movement, analog input, digital output, and event logic for the FMC920. |
| Full Parameter List | wiki.teltonika-gps.com/view/FMC920_Parameter_list | All configurable parameters for the FMC920. |

## 6.3  Protocol & Developer Documentation

These are the core technical documents your backend developer must read before writing a single line of parser code.

| Document | Direct URL | Why It Matters |
| --- | --- | --- |
| Codec Reference (8, 8E, 12, 14, 15) | wiki.teltonika-gps.com/view/Codec | Full specification for every Teltonika binary protocol. Codec 8 Extended (0x8E) is what FMC130 uses for all CAN/advanced data. Includes hex packet examples with worked decoding. |
| Data Sending Protocols | wiki.teltonika-gps.com/view/Teltonika_Data_Sending_Protocols | TCP/UDP socket behaviour, IMEI handshake sequence, acknowledgement format, SMS binary protocol, 24-hour compressed SMS format. |
| FMC130 AVL Parameter IDs | wiki.teltonika-gps.com/view/FMC130_Teltonika_Data_Sending_Parameters_ID | The lookup table your parser uses to translate a raw 2-byte ID into 'Engine RPM' or 'Fuel Level %'. Updated to firmware FMB.Ver.03.29.00+. |
| FMC920 AVL Parameter IDs | wiki.teltonika-gps.com/view/FMC920_Teltonika_Data_Sending_Parameters_ID | Same as above but for the FMC920's parameter set. |
| J1939 / OBD to FMS Adapter Guide | wiki.teltonika-gps.com/view/OBD_II_TO_FMS_ADAPTER_AND_J1939_SUPPORT_IN_FMx003 | How to connect Teltonika OBD devices to heavy duty vehicles via the FMS connector for J1939 data. Directly applicable to heavy equipment CAN setup. |

## 6.4  Tools, Software & Platform Downloads

| Tool | URL | What It Does |
| --- | --- | --- |
| Teltonika Configurator (free) | wiki.teltonika-gps.com/view/Teltonika_Configurator | Windows desktop app to configure any Teltonika device via USB or Bluetooth. Set server IP, tracking intervals, I/O logic, CAN parameters. Download from this wiki page. |
| Configurator Version History | wiki.teltonika-gps.com/view/Teltonika_Configurator_versions | Always check this before downloading — get the version that matches your device firmware. |
| FOTA WEB (remote management) | wiki.teltonika-gps.com/view/FOTA_WEB | Teltonika's cloud platform for over-the-air firmware updates and config push across entire device fleet. Available to direct buyers. |
| COM Port USB Drivers | Linked from FMC130 and FMC920 wiki pages | Required for Windows to recognize the device via Micro-USB. Install before using Configurator. |
| Teltonika Wiki Main Page | wiki.teltonika-gps.com/view/Main_Page | Master index. Use the search bar or browse by device family. Every document in this briefing is accessible from here. |

## 6.5  Community & Support

| Resource | URL | What It's For |
| --- | --- | --- |
| Teltonika Community Forum | community.teltonika.lt | The most useful support resource after the wiki. Real installers post real problems. Search before raising issues — most CAN mapping questions, wiring problems, and server configuration issues already have answers here. |
| FMC130 Protocol Discussion Thread | community.teltonika.lt (search 'FMC130 protocol') | Community threads on building custom parsers, server setup, and CAN adapter configuration. |
| Flespi Device API (FMC130) | flespi.com/devices/teltonika-fmc130 | Commercial telematics hub that parses FMC130 data and serves standardized JSON via REST API. Good alternative to building a custom parser — paid service per device/month. |
| Traccar Open Source Backend | github.com/traccar/traccar / traccar.org/documentation/ | Free, production-grade telematics backend with full Teltonika protocol support built in. Recommended Phase 1 deployment. |

Telematics Technical Briefing  |  Internal Confidential Document  |  April 2026